Automated E-Waste Disposal System

CNN-Based Classification & Sorting

Full Implementation Plan

Based on: IOP Conf. Ser.: Earth Environ. Sci. 1529 (2025) 012032

Dataset: Roboflow e-waste-dataset-r0ojc (v1)

Fixes: Class Imbalance | GPU-Forced Training | Training Stability

# 0. Project Overview & Paper Analysis

This implementation plan is derived entirely from the research paper by Kondaveeti et al. (2025), cross-referenced with the Roboflow e-waste dataset, and extended to fix the two confirmed implementation problems: (1) training on an imbalanced dataset and (2) training stopping prematurely due to CPU bottleneck.

The paper proposes a Raspberry Pi 4 + CNN-based automated e-waste sorter that:

- Captures images via camera triggered by an ultrasonic sensor

- Classifies e-waste into 10 categories using a CNN model

- Sorts items into bins via a servo motor controlled through GPIO

- Achieves 95% training accuracy and 93% validation accuracy

- Processes each item in 2-3 seconds end-to-end

# 1. Hardware Components (from Paper Section 2.6-2.8)

Every hardware component listed in the paper must be present for a complete build:

| Component | Specification (Paper) | Role in System |
| --- | --- | --- |
| Raspberry Pi 4 | Broadcom BCM2711, Cortex-A72 quad-core, up to 8GB LPDDR4, microSD storage, 5V USB-C | Central processing unit: runs CNN inference, controls GPIO, stores data locally |
| Camera Module | Pi Camera (standard) | Captures image of e-waste item placed on drop-off bin |
| Ultrasonic Sensor | 5V DC, 2-400 cm range, ±1 cm accuracy | Detects presence of object in bin; triggers camera capture |
| Servo Motor | 1.5-30 kg·cm torque, 0.15-0.2 sec/60°, 4.8V-6V, 0°-180° rotation | Moves sorting gate to direct item into correct bin |
| Bins | Two bins minimum (e-waste / non-e-waste) | Physical separation containers for sorted items |
| Display | Any HDMI/I2C display | Shows e-waste category to user for awareness |

## 1.1 Wiring (from Paper Figure 5 - Schematic Diagram)

Based on Figure 5 of the paper:

- Ultrasonic Sensor → VCC, ECHO, TRIG, GND pins of Raspberry Pi GPIO

- Servo Motor → GPIO pin 6 on Raspberry Pi board

- Camera Module → CSI camera port on Raspberry Pi

- Power supply feeds both the Pi and servo motor (servo needs 4.8V-6V; use external supply for servo to avoid GPIO current limits)

# 2. Dataset Setup (Paper Section 2.1-2.2 + Roboflow Source)

## 2.1 Paper Dataset Specification

The paper specifies the following dataset structure exactly:

| Parameter | Paper Value |
| --- | --- |
| Total training images | 2,400 images |
| Class distribution (training) | Equally distributed across 10 classes (240 per class) |
| Validation images | 300 images (proportional split) |
| Number of classes | 10 |
| Image resolution (input) | 180 × 180 pixels |
| Batch size | 32 images |
| Pixel normalization | [0,255] → [0,1] via Rescaling(1./255) |

## 2.2 Paper's 10 E-Waste Categories (Section 2.2)

The following 10 classes are explicitly named in the paper:

- Battery

- Keyboard

- Microwave

- Mobile

- Mouse

- PCB (Printed Circuit Board)

- Player

- Printer

- Television

- Washing Machine

## 2.3 Roboflow Dataset (e-waste-dataset-r0ojc v1)

The Roboflow dataset (e-waste-dataset-r0ojc) covers UNU-KEY e-waste categories and contains 19,612 annotated images at 640x640 resolution. It covers 22+ categories aligned with the UNU-KEY classification standard. Since this is an object detection dataset (bounding box annotations), you will need to convert it to a classification format.

⚠ FIX: The Roboflow dataset is imbalanced by nature (different categories have vastly different numbers of images). This is Problem #1 you encountered. The fix is described in full in Section 4.

## 2.4 Adapting Roboflow Dataset to Match Paper

Step-by-step adaptation process:

- Download dataset from Roboflow in folder/classification format or convert from YOLO format

- Map Roboflow classes to the 10 paper categories. Classes not matching any of the 10 categories should be discarded or grouped

- After mapping, apply class balancing (Section 4 below) to achieve 240 images per class in training set

- Resize all images to 180x180 as specified in the paper

- Split: 2400 train / 300 validation / 300 test (or adjust proportionally)

# 3. Data Preprocessing & Augmentation (Paper Section 2.3)

The paper is explicit about every preprocessing step. Implement all of them exactly:

## 3.1 Core Preprocessing

- Resize all images to 180 × 180 pixels

- Batch size: 32 images

- Shuffle training and validation sets (to reduce bias, as stated in paper)

- Normalize pixel values from [0, 255] to [0, 1] using Rescaling(1./255)

- Organize images into subdirectories per class label (enables automatic labeling on load)

## 3.2 Data Augmentation (Training Only)

The paper lists these augmentation techniques explicitly — apply all of them during training to reduce overfitting:

- Random flipping: both horizontal and vertical

- Random rotation: within ±20 degrees

- Random zooming

- Random contrast adjustment

- Random translation: horizontal and vertical shifts

NOTE: Augmentation is applied ONLY during training, not validation or inference. This matches standard practice and the paper's intent.

# 4. Fix #1 — Class Imbalance Solution

⚠ FIX: Your earlier code trained on an imbalanced dataset. This biases the model toward majority classes and explains any accuracy inconsistency across categories.

Three complementary strategies must all be applied:

## 4.1 Strategy A: Undersample Majority Classes

Cap the number of images per class to a fixed maximum (240 per class, matching the paper's 2400 total training images across 10 classes). This prevents any single class from dominating gradient updates.

# In your dataset loading script:

MAX_PER_CLASS = 240  # Paper specifies 240 images/class (2400 total / 10 classes)

from collections import defaultdict

import random

def balanced_file_list(root_dir, max_per_class=MAX_PER_CLASS):

class_files = defaultdict(list)

for class_name in os.listdir(root_dir):

class_path = os.path.join(root_dir, class_name)

if os.path.isdir(class_path):

files = [os.path.join(class_path, f) for f in os.listdir(class_path)]

random.shuffle(files)

class_files[class_name] = files[:max_per_class]  # Cap at max

return class_files

## 4.2 Strategy B: Class Weights in Loss Function

Even after undersampling, use class weights in the model's loss function. This makes the optimizer penalize errors on minority classes more heavily during any remaining imbalance.

from sklearn.utils.class_weight import compute_class_weight

import numpy as np

# Compute class weights from training labels

class_weights = compute_class_weight(

class_weight='balanced',

classes=np.unique(train_labels),

y=train_labels

)

class_weight_dict = dict(enumerate(class_weights))

# Pass to model.fit()

model.fit(

train_dataset,

validation_data=val_dataset,

epochs=10,  # Paper trains for 10 epochs

class_weight=class_weight_dict,  # <-- KEY FIX

callbacks=[...],

)

## 4.3 Strategy C: Augment Minority Classes (Oversampling)

For classes that have fewer than 240 images in the Roboflow dataset, apply heavier augmentation to synthesize additional samples, reaching 240 images per class. This matches the paper's equal distribution across all 10 classes.

import tensorflow as tf

def augment_image(image):

# Exact augmentations from paper Section 2.3

image = tf.image.random_flip_left_right(image)

image = tf.image.random_flip_up_down(image)        # Paper: horizontal & vertical flip

image = tf.keras.layers.RandomRotation(0.055)(image)  # +-20 degrees = 0.055 of 2pi

image = tf.keras.layers.RandomZoom(0.1)(image)

image = tf.image.random_contrast(image, 0.8, 1.2)

image = tf.keras.layers.RandomTranslation(0.1, 0.1)(image)

return image

# For each minority class with N < MAX_PER_CLASS images,

# apply augment_image() repeatedly until len == MAX_PER_CLASS

# 5. Fix #2 — Force GPU Training (Prevent Training from Stopping)

⚠ FIX: Training was running on CPU and stopping before completion. GPU must be explicitly forced and verified before training begins.

## 5.1 Verify GPU is Available

import tensorflow as tf

# Step 1: Check GPUs

gpus = tf.config.list_physical_devices('GPU')

print('GPUs available:', gpus)

if not gpus:

raise RuntimeError('ERROR: No GPU found. Training cannot proceed on CPU.')

# If this raises: install CUDA + cuDNN matching your TF version,

# or use Google Colab (free GPU) or a cloud VM with GPU.

# Step 2: Enable memory growth (prevents TF from grabbing all VRAM at once)

for gpu in gpus:

tf.config.experimental.set_memory_growth(gpu, True)

print('GPU setup complete. Training will use:', gpus[0].name)

## 5.2 Force GPU Usage with tf.device Context

# Wrap model building AND training inside GPU context

with tf.device('/GPU:0'):

model = build_model()      # Model construction on GPU

model.fit(                 # Training on GPU

train_dataset,

validation_data=val_dataset,

epochs=10,

class_weight=class_weight_dict,

callbacks=get_callbacks(),

)

## 5.3 Add Callbacks to Prevent Stopping & Save Progress

These callbacks prevent training from stopping prematurely and save checkpoints so you can resume:

def get_callbacks():

return [

# Save best model automatically (do not lose progress)

tf.keras.callbacks.ModelCheckpoint(

filepath='best_model.keras',

monitor='val_accuracy',

save_best_only=True,

verbose=1

),

# Stop only if truly no improvement for 5 epochs (not prematurely)

tf.keras.callbacks.EarlyStopping(

monitor='val_accuracy',

patience=5,          # Wait 5 epochs before stopping

restore_best_weights=True,

verbose=1

),

# Log progress for analysis

tf.keras.callbacks.CSVLogger('training_log.csv'),

# Reduce LR if stuck

tf.keras.callbacks.ReduceLROnPlateau(

monitor='val_loss',

factor=0.5,

patience=3,

verbose=1

),

]

## 5.4 Environment Options for GPU Training

If you don't have a local GPU:

| Option | GPU Available | Cost | Recommended For |
| --- | --- | --- | --- |
| Google Colab (free tier) | T4 / V100 (shared) | Free | Development, testing |
| Google Colab Pro | A100 | ~$10/month | Full training runs |
| Kaggle Kernels | T4 x2 | Free (30hr/week) | Dataset already on Kaggle |
| Local NVIDIA GPU | Your GPU | One-time hardware cost | Production deployment |
| AWS/GCP VM (GPU instance) | A10/V100/A100 | Pay-per-use | Industrial scale |

# 6. CNN Model Architecture (Paper Section 2.4-2.5)

The paper specifies the complete CNN architecture. Implement it exactly as described:

## 6.1 Architecture Specification (from Paper Table/Text)

| Layer | Details (from Paper) | Purpose |
| --- | --- | --- |
| Input | 180 × 180 × 3 (RGB) | Accepts camera images resized to 180x180 |
| Rescaling | 1./255 normalization | Normalizes [0,255] to [0,1] |
| Data Augmentation | Flip, rotate ±20°, zoom, contrast, translate | Applied during training only |
| Conv2D (1) | 16 filters, ReLU activation | Detects low-level features (edges, textures) |
| MaxPooling2D (1) | Reduces spatial dimensions | Downsamples feature maps |
| Conv2D (2) | 32 filters, ReLU activation | Intermediate feature detection |
| MaxPooling2D (2) | Reduces spatial dimensions | Downsampling |
| Conv2D (3) | 64 filters, ReLU activation | High-level feature detection |
| MaxPooling2D (3) | Reduces spatial dimensions | Downsampling |
| Flatten | Converts 3D feature map to 1D | Prepares for dense layers |
| Dense | 128 units, ReLU activation | Fully connected classification layer |
| Dropout | Rate = 0.2 | Prevents overfitting (paper specifies 0.2) |
| Output Dense | 10 units, softmax activation | One output per e-waste category, produces probabilities |

## 6.2 Complete Model Code

import tensorflow as tf

from tensorflow import keras

from tensorflow.keras import layers

NUM_CLASSES = 10   # Paper: 10 e-waste categories

IMG_SIZE = 180     # Paper: 180x180 pixels

def build_ewaste_cnn():

# Data augmentation block (paper Section 2.3)

data_augmentation = keras.Sequential([

layers.RandomFlip('horizontal_and_vertical'),   # Paper: horizontal & vertical

layers.RandomRotation(0.055),                   # Paper: +-20 degrees

layers.RandomZoom(0.1),                         # Paper: random zooming

layers.RandomContrast(0.2),                     # Paper: random contrast

layers.RandomTranslation(0.1, 0.1),             # Paper: horizontal/vertical shifts

])

model = keras.Sequential([

keras.Input(shape=(IMG_SIZE, IMG_SIZE, 3)),

layers.Rescaling(1./255),                       # Paper: normalize [0,255]->[0,1]

data_augmentation,                              # Paper: augmentation during training

# Convolutional blocks (paper: 16, 32, 64 filters)

layers.Conv2D(16, (3,3), activation='relu', padding='same'),

layers.MaxPooling2D(),

layers.Conv2D(32, (3,3), activation='relu', padding='same'),

layers.MaxPooling2D(),

layers.Conv2D(64, (3,3), activation='relu', padding='same'),

layers.MaxPooling2D(),

layers.Flatten(),

layers.Dense(128, activation='relu'),           # Paper: 128 units

layers.Dropout(0.2),                           # Paper: dropout rate 0.2

layers.Dense(NUM_CLASSES, activation='softmax') # Paper: softmax, 10 classes

])

# Paper: Adam optimizer, sparse categorical cross-entropy

model.compile(

optimizer='adam',

loss='sparse_categorical_crossentropy',

metrics=['accuracy']

)

return model

# 7. Training Configuration (Paper Section 2.5 + Fixes)

| Parameter | Paper Value | Fix Applied |
| --- | --- | --- |
| Optimizer | Adam | No change needed |
| Loss function | Sparse categorical cross-entropy (multiclass) | Add class_weight parameter |
| Batch size | 32 images | No change needed |
| Epochs | 10 | Add EarlyStopping with patience=5 to prevent overrun OR premature stop |
| Training set | 2,400 images (240/class, balanced) | Apply undersampling + oversampling fix |
| Validation set | 300 images | No change needed |
| Device | GPU (forced) | Add tf.device('/GPU:0') and GPU verification check |
| Metrics | Accuracy and Loss | Add CSVLogger to track both per epoch |

# 8. Evaluation & Results (Paper Section 4)

The paper evaluates on the following metrics. Your implementation must reproduce each:

| Metric | Paper Result | How to Reproduce |
| --- | --- | --- |
| Training Accuracy | 95% | model.fit() — history['accuracy'] |
| Validation Accuracy | 93% | model.fit() — history['val_accuracy'] |
| Training Loss | Continuously decreasing | Plot history['loss'] — must show downward trend |
| Validation Loss | Downward trend throughout training | Plot history['val_loss'] |
| Confusion Matrix | 171 TP, 129 FP, 129 FN, 2331 TN (cumulative) | Use sklearn.metrics.confusion_matrix |
| Prototype Sorting Accuracy | 93% | Physical hardware test — 2-3 seconds per item |

## 8.1 Confusion Matrix Code (Paper Section 4.5)

from sklearn.metrics import confusion_matrix, classification_report

import matplotlib.pyplot as plt

import seaborn as sns

# Evaluate on test set

y_pred = model.predict(test_dataset)

y_pred_classes = np.argmax(y_pred, axis=1)

# Confusion matrix (paper Figure 8 format)

cm = confusion_matrix(y_true, y_pred_classes)

plt.figure(figsize=(10,8))

sns.heatmap(cm, annot=True, fmt='d', cmap='Blues',

xticklabels=CLASS_NAMES, yticklabels=CLASS_NAMES)

plt.title('E-waste Classification Confusion Matrix')

plt.savefig('confusion_matrix.png')

# Also print classification report per class

print(classification_report(y_true, y_pred_classes, target_names=CLASS_NAMES))

# 9. Raspberry Pi Deployment (Paper Section 2.6-3.2)

After training on GPU (PC/cloud), export the model and run inference on Raspberry Pi 4. The Pi is not used for training — only for inference and GPIO control.

## 9.1 Model Export for Raspberry Pi

# On training machine (after training completes):

model.save('ewaste_cnn_model.keras')         # Full model save

# Convert to TFLite for faster inference on Pi (recommended)

converter = tf.lite.TFLiteConverter.from_keras_model(model)

tflite_model = converter.convert()

with open('ewaste_model.tflite', 'wb') as f:

f.write(tflite_model)

# Transfer ewaste_model.tflite to Raspberry Pi via scp or USB

## 9.2 Raspberry Pi Inference + GPIO Control

This is the main runtime loop that runs on the Pi. It implements the full pipeline from paper Figure 2 (Block Diagram):

# Pi Runtime: ewaste_sorter.py

import RPi.GPIO as GPIO

import time

import numpy as np

from picamera2 import Picamera2

import tflite_runtime.interpreter as tflite

from PIL import Image

# GPIO Pin setup (from paper Figure 5 schematic)

TRIG_PIN = 23        # Ultrasonic trigger

ECHO_PIN = 24        # Ultrasonic echo

SERVO_PIN = 6        # Servo motor (paper: GPIO pin 6)

GPIO.setmode(GPIO.BCM)

GPIO.setup(TRIG_PIN, GPIO.OUT)

GPIO.setup(ECHO_PIN, GPIO.IN)

GPIO.setup(SERVO_PIN, GPIO.OUT)

servo = GPIO.PWM(SERVO_PIN, 50)   # 50Hz for servo

servo.start(0)

CLASS_NAMES = ['Battery','Keyboard','Microwave','Mobile','Mouse',

'PCB','Player','Printer','Television','WashingMachine']

EWASTE_CLASSES = list(range(10))  # All 10 are e-waste

# Adjust above if you have a non-e-waste category

# Load TFLite model

interpreter = tflite.Interpreter(model_path='ewaste_model.tflite')

interpreter.allocate_tensors()

input_details = interpreter.get_input_details()

output_details = interpreter.get_output_details()

camera = Picamera2()

camera.start()

def measure_distance():

GPIO.output(TRIG_PIN, True)

time.sleep(0.00001)

GPIO.output(TRIG_PIN, False)

start = time.time()

while GPIO.input(ECHO_PIN) == 0: start = time.time()

while GPIO.input(ECHO_PIN) == 1: stop = time.time()

return (stop - start) * 34300 / 2   # Distance in cm

def classify_image(img_array):

img_array = img_array.astype(np.float32)

interpreter.set_tensor(input_details[0]['index'], [img_array])

interpreter.invoke()

output = interpreter.get_tensor(output_details[0]['index'])

return np.argmax(output), np.max(output)

def sort_item(is_ewaste):

# Servo: 90 deg = e-waste bin, 0 deg = non-e-waste bin

angle = 90 if is_ewaste else 0

duty = angle / 18 + 2

servo.ChangeDutyCycle(duty)

time.sleep(0.5)

servo.ChangeDutyCycle(0)

print('E-Waste Sorter Ready. Waiting for objects...')

try:

while True:

dist = measure_distance()

if dist < 20:    # Object detected within 20 cm

print(f'Object detected at {dist:.1f} cm. Capturing image...')

time.sleep(0.2)

frame = camera.capture_array()

img = Image.fromarray(frame).resize((180, 180))

img_array = np.array(img) / 255.0

class_idx, confidence = classify_image(img_array)

label = CLASS_NAMES[class_idx]

print(f'Classified as: {label} (confidence: {confidence:.2%})')

sort_item(is_ewaste=True)  # All 10 classes are e-waste in this setup

time.sleep(2)    # Wait before next detection (paper: 2-3 sec per item)

time.sleep(0.1)

except KeyboardInterrupt:

servo.stop()

GPIO.cleanup()

camera.stop()

# 10. Data Management (Paper Section 3.2)

The paper specifies that ALL data management is done locally on Raspberry Pi — no cloud storage. This is a design decision embedded in the paper:

- Raspberry Pi OS is the operating system on the Pi

- MicroSD card is the primary storage medium

- The Pi acts as the central processing unit for data collection and model inference

- The memory card stores both the operating system and all processed data

- No cloud dependency — system is self-contained and autonomous

- This means: train on PC/cloud, transfer model file to Pi's microSD, run everything locally on Pi

NOTE: Use at least a 32GB Class 10 microSD card. Store training logs, captured images, and classification results in organized directories on the card.

# 11. Scalability (Paper Section 4.7)

The paper explicitly lists future scalability upgrades. Implement these if expanding beyond the prototype:

| Upgrade | Paper's Recommendation | Implementation Note |
| --- | --- | --- |
| Processing Power | Faster processors | Use Raspberry Pi 5, or Jetson Nano for edge ML |
| Camera Quality | High-resolution cameras | Use Pi HQ Camera (12MP) for better classification |
| Continuous Sorting | Add conveyor belt | Integrate stepper motor + conveyor for industrial use |
| Cloud Storage | Store data on cloud | Add MQTT or HTTP upload to AWS/GCP when Wi-Fi available |
| Real-time Monitoring | Track performance & improvements | Dashboard using Flask + Grafana on local network |
| Expanded Dataset | Train on more e-waste types | Add more UNU-KEY classes from Roboflow dataset |
| Industrial Deployment | Scale for recycling centers | Multiple Pi units + central server for coordination |

# 12. Complete Training Script (All Fixes Integrated)

This is the full training script with all paper specifications and both fixes applied. Run this on a machine with GPU:

# train_ewaste_cnn.py — Full training script

# Based on: IOP Conf. Ser.: Earth Environ. Sci. 1529 (2025) 012032

# Fixes: Class imbalance + GPU-forced training

import os, random, numpy as np

import tensorflow as tf

from tensorflow import keras

from tensorflow.keras import layers

from sklearn.utils.class_weight import compute_class_weight

from collections import defaultdict

# ── GPU SETUP (Fix #2) ──────────────────────────────────────

gpus = tf.config.list_physical_devices('GPU')

if not gpus:

raise RuntimeError('No GPU found. Install CUDA or use Colab.')

for gpu in gpus:

tf.config.experimental.set_memory_growth(gpu, True)

print(f'Training on GPU: {gpus[0].name}')

# ── CONFIG (from Paper) ─────────────────────────────────────

IMG_SIZE   = 180       # Paper Section 2.3

BATCH_SIZE = 32        # Paper Section 2.3

EPOCHS     = 10        # Paper Section 2.5

NUM_CLASSES= 10        # Paper Section 2.2

MAX_PER_CLASS = 240    # Paper: 2400 train / 10 classes

DATASET_DIR = './dataset'   # Adjust to your path

CLASS_NAMES = ['Battery','Keyboard','Microwave','Mobile','Mouse',

'PCB','Player','Printer','Television','WashingMachine']

# ── BALANCED DATASET LOADER (Fix #1) ────────────────────────

def load_balanced_dataset(split_dir, max_per_class):

all_paths, all_labels = [], []

for idx, cls in enumerate(CLASS_NAMES):

cls_dir = os.path.join(split_dir, cls)

if not os.path.isdir(cls_dir): continue

files = [os.path.join(cls_dir, f) for f in os.listdir(cls_dir)

if f.lower().endswith(('.jpg','.jpeg','.png'))]

random.shuffle(files)

files = files[:max_per_class]  # Cap per class (fix imbalance)

all_paths.extend(files)

all_labels.extend([idx] * len(files))

return all_paths, all_labels

def preprocess_image(path, label):

img = tf.io.read_file(path)

img = tf.image.decode_jpeg(img, channels=3)

img = tf.image.resize(img, [IMG_SIZE, IMG_SIZE])

return img, label

train_paths, train_labels = load_balanced_dataset(

os.path.join(DATASET_DIR, 'train'), MAX_PER_CLASS)

val_paths, val_labels   = load_balanced_dataset(

os.path.join(DATASET_DIR, 'valid'), MAX_PER_CLASS // 4)

# Class weights (Fix #1 — second layer of protection)

cw = compute_class_weight('balanced',

classes=np.unique(train_labels), y=train_labels)

class_weight_dict = dict(enumerate(cw))

# Build tf.data pipelines

AUTOTUNE = tf.data.AUTOTUNE

train_ds = (tf.data.Dataset.from_tensor_slices((train_paths, train_labels))

.shuffle(len(train_paths))

.map(preprocess_image, num_parallel_calls=AUTOTUNE)

.batch(BATCH_SIZE)

.prefetch(AUTOTUNE))

val_ds = (tf.data.Dataset.from_tensor_slices((val_paths, val_labels))

.map(preprocess_image, num_parallel_calls=AUTOTUNE)

.batch(BATCH_SIZE)

.prefetch(AUTOTUNE))

# ── MODEL (from Paper Section 2.5) ──────────────────────────

with tf.device('/GPU:0'):   # Fix #2: force GPU

data_aug = keras.Sequential([

layers.RandomFlip('horizontal_and_vertical'),

layers.RandomRotation(0.055),

layers.RandomZoom(0.1),

layers.RandomContrast(0.2),

layers.RandomTranslation(0.1, 0.1),

])

model = keras.Sequential([

keras.Input(shape=(IMG_SIZE, IMG_SIZE, 3)),

layers.Rescaling(1./255),

data_aug,

layers.Conv2D(16, 3, activation='relu', padding='same'),

layers.MaxPooling2D(),

layers.Conv2D(32, 3, activation='relu', padding='same'),

layers.MaxPooling2D(),

layers.Conv2D(64, 3, activation='relu', padding='same'),

layers.MaxPooling2D(),

layers.Flatten(),

layers.Dense(128, activation='relu'),

layers.Dropout(0.2),

layers.Dense(NUM_CLASSES, activation='softmax'),

])

model.compile(optimizer='adam',

loss='sparse_categorical_crossentropy',

metrics=['accuracy'])

model.summary()

callbacks = [

keras.callbacks.ModelCheckpoint(

'best_ewaste_model.keras', monitor='val_accuracy',

save_best_only=True, verbose=1),

keras.callbacks.EarlyStopping(

monitor='val_accuracy', patience=5,

restore_best_weights=True, verbose=1),

keras.callbacks.CSVLogger('training_log.csv'),

keras.callbacks.ReduceLROnPlateau(

monitor='val_loss', factor=0.5, patience=3, verbose=1),

]

history = model.fit(

train_ds,

validation_data=val_ds,

epochs=EPOCHS,

class_weight=class_weight_dict,  # Fix #1

callbacks=callbacks,

)

# Save final model + TFLite export

model.save('ewaste_cnn_final.keras')

converter = tf.lite.TFLiteConverter.from_keras_model(model)

with open('ewaste_model.tflite', 'wb') as f:

f.write(converter.convert())

print('Training complete. Models saved.')

# 13. Implementation Checklist

Use this checklist to track completion of every paper requirement:

## Hardware

- Raspberry Pi 4 (Broadcom BCM2711, Cortex-A72, up to 8GB RAM)

- Pi Camera Module connected to CSI port

- Ultrasonic sensor wired to GPIO (VCC, ECHO, TRIG, GND)

- Servo motor wired to GPIO pin 6, external 4.8-6V supply

- Two sorting bins labeled (e-waste / non-e-waste)

- Display for showing classification result to user

## Dataset

- Roboflow dataset (e-waste-dataset-r0ojc v1) downloaded

- Classes mapped to the 10 paper categories

- Dataset balanced to 240 images per class in training set

- Validation set: ~30 images per class

- Images resized to 180x180 pixels

- Organized in subdirectories per class label

## Training (with Fixes)

- GPU verified available before training starts

- Training wrapped in tf.device('/GPU:0')

- Class weights computed and passed to model.fit()

- Undersampling applied (max 240 images per class)

- All 5 augmentation techniques from paper applied

- ModelCheckpoint callback saving best model

- EarlyStopping with patience=5 (not premature)

- CSVLogger saving epoch-by-epoch metrics

- Adam optimizer, sparse categorical cross-entropy, batch=32, 10 epochs

## Model Architecture

- Conv2D: 16 filters → MaxPool

- Conv2D: 32 filters → MaxPool

- Conv2D: 64 filters → MaxPool

- Flatten → Dense(128) → Dropout(0.2) → Dense(10, softmax)

## Evaluation

- Training accuracy plotted (target: 95%)

- Validation accuracy plotted (target: 93%)

- Training loss curve plotted (must be decreasing)

- Validation loss curve plotted (must be decreasing)

- Confusion matrix generated for all 10 classes

- Classification report (precision, recall, F1) per class

## Deployment

- Model exported to TFLite format

- TFLite model transferred to Raspberry Pi microSD

- Inference loop running: sensor → camera → classify → servo → display

- Physical sorting verified at 2-3 seconds per item

Implementation Plan v1.0 | Based on IOP Conf. Ser.: Earth Environ. Sci. 1529 (2025) 012032